[image: image1.png]=
P
¥
v
g
3/
= s
a0
%
il





Dlaczego poniższy program mi nie działa?
0  BEGIN PGM frez_cc2 MM 

1  BLK FORM 0.1 Z  X-26  Y-22  Z-20

2  BLK FORM 0.2  X+26  Y+22  Z+0

3  TOOL CALL 6 Z S2000 F300

4  ;frez palcowy fi12

5  L  Z+200 R0 FMAX

6  L  X-36  Y-10 R0 FMAX M3

7  CYCL DEF 14.0 CONTOUR GEOMETRY

8  CYCL DEF 14.1 CONTOUR LABEL1

9  CYCL DEF 25 CONTOUR TRAIN ~

    Q1=-11   ;MILLING DEPTH ~

    Q3=+0    ;ALLOWANCE FOR SIDE ~

    Q5=+0    ;SURFACE COORDINATE ~

    Q7=+50   ;CLEARANCE HEIGHT ~

    Q10=-5    ;PLUNGING DEPTH ~

    Q11=+150  ;FEED RATE FOR PLNGNG ~

    Q12=+500  ;FEED RATE F. ROUGHNG ~

    Q15=+1    ;CLIMB OR UP-CUT

10 CYCL CALL M3

11 L  Z+200 R0 FMAX

12 TOOL CALL 7 Z S3000 F300

13 ;frez palcowy do rowkow fi12

14 CYCL DEF 251 RECTANGULAR POCKET ~

    Q215=+0    ;MACHINING OPERATION ~

    Q218=+20   ;FIRST SIDE LENGTH ~

    Q219=+20   ;SECOND SIDE LENGTH ~

    Q220=+5    ;CORNER RADIUS ~

    Q368=+0    ;ALLOWANCE FOR SIDE ~

    Q224=+0    ;ANGLE OF ROTATION ~

    Q367=+0    ;POCKET POSITION ~

    Q207=+500  ;FEED RATE FOR MILLNG ~

    Q351=+1    ;CLIMB OR UP-CUT ~

    Q201=-9    ;DEPTH ~

    Q202=+3    ;PLUNGING DEPTH ~

    Q369=+0    ;ALLOWANCE FOR FLOOR ~

    Q206=+150  ;FEED RATE FOR PLNGNG ~

    Q338=+0    ;INFEED FOR FINISHING ~

    Q200=+2    ;SET-UP CLEARANCE ~

    Q203=+0    ;SURFACE COORDINATE ~

    Q204=+50   ;2ND SET-UP CLEARANCE ~

    Q370=+1    ;TOOL PATH OVERLAP ~

    Q366=+1    ;PLUNGE ~

    Q385=+500  ;FINISHING FEED RATE

15 CYCL CALL M3

16 L  Z+200 R0 FMAX

17 TOOL CALL 8 Z S300 F300

18 ;wiertlo fi4.8

19 CYCL DEF 200 DRILLING ~

    Q200=+10   ;SET-UP CLEARANCE ~

    Q201=-21.2241 ;DEPTH ~

    Q206=+150  ;FEED RATE FOR PLNGNG ~

    Q202=+4    ;PLUNGING DEPTH ~

    Q210=+0    ;DWELL TIME AT TOP ~

    Q203=+0    ;SURFACE COORDINATE ~

    Q204=+50   ;2ND SET-UP CLEARANCE ~

    Q211=+0    ;DWELL TIME AT DEPTH

20 CYCL DEF 220 POLAR PATTERN ~

    Q216=+26   ;CENTER IN 1ST AXIS ~

    Q217=+22   ;CENTER IN 2ND AXIS ~

    Q244=+32   ;PITCH CIRCLE DIAMETR ~

    Q245=+60   ;STARTING ANGLE ~

    Q246=+360  ;STOPPING ANGLE ~

    Q247=+60   ;STEPPING ANGLE ~

    Q241=+6    ;NR OF REPETITIONS ~

    Q200=+10   ;SET-UP CLEARANCE ~

    Q203=+0    ;SURFACE COORDINATE ~

    Q204=+50   ;2ND SET-UP CLEARANCE ~

    Q301=+1    ;MOVE TO CLEARANCE ~

    Q365=+0    ;TYPE OF TRAVERSE

21 CYCL CALL M3

22 L  Z+200 R0 FMAX

23 STOP M30

24 LBL 1

25 APPR LCT  X-26  Y+0 R10 FMAX

26 CR  X-23  Y+11 R+20.6 DR-

27 CR  X-15  Y+19 R+8 DR+

28 CR  X+15  Y+19 R+70.48 DR+

29 CR  X+23  Y+11 R+8 DR+

30 CR  X+23  Y-11 R+20.6 DR-

31 CR  X+15  Y-19 R+8 DR+

32 CR  X-15  Y-19 R+70.48 DR+

33 CR  X-23  Y-11 R+8 DR+

34 CR  X-26  Y+0 R+20.6 DR-

35 DEP LN LEN+20 FMAX

36 LBL 0

37 END PGM frez_cc2 MM 

